Advanced Computer Architecture

Fall 2005

Homework #5

Due at 6:15PM, Tuesday, November 29, 2005

(email to aberenbaum@acm.org by 6:15PM is acceptable)

(1)

In the following loop:

LOOP:	LW		R4,0(R8)

LW		R5,0(R9)

DSLL		R1,R4,#2

DADD		R2,R5,R1

DADDI	R2,R2,#10

SW		R2,400(R8)

DADDI	R8,R8,#4

DADDI	R9,R9,#4

DADDI	R10,R10,#-1

BNEZ		R10,LOOP

assume that all loads require a 1-cycle stall and all conditional branches cause a 2-cycle stall. Assume full forwarding and splitting for data hazards.

- What is the average CPI for the above loop?

- Rewrite the loop by unrolling the loop one time, so that the loop body contains two iterations of the loop.

- What is the CPI of the rewritten loop?

(2)

Assume an architecture that supports predication on every instruction by adding an optional predicate based on a 1-bit condition code. For the C code snippet:

z = 2;

if(x > 0) {

y = x + 2;

} else {

z = 3;

y = x – 4;

}

This code can be compiled into a pseudo-machine as follows:

mov	$r1 = 2			# z in $r1

cmpgt 	$b0 = $r2, 0	# x in $r2, condition in $b0

br $b0, L1

add $r3 = $r2, 2		# y in $r2

goto L2:

L1:

mov $r1 = 3

add $r3, $r2, -4

L2:

…

Assume a 4-wide VLIW machine with a fixed 4-wide issue that can issue any 4 operations in one cycle. Rewrite the code without any branches and schedule on the VLIW.

(2) Hennessy/Patterson 4.8

Note: For both parts (a) and (b), you may assume that the number of iterations will be a multiple of the number of copies you make by loop unrolling (i.e. you do not have to have a second startup loop to take care of the remainder elements)

(3) Hennessy/Patterson 4.13